# Leakage rate prediction through cracked concrete elements:

# A literature review (supplementary material)

Liam Maa, Oh-Sung Kwona,, Evan C. Bentza,

a Department of Civil & Mineral Engineering, University of Toronto, 35 St. George Street, Toronto, Canada

Information

This document contains supplementary material for the literature review paper. A summary of the notation used for this paper and supplementary document is presented in the section titled “Notation”. Sample calculations for leakage rates determined from select models are found in the section titled “Leakage rate prediction calculations”.

Notation

Latin symbols:

a	=	coefficient of air leakage

a	=	empirical relative roughness expression

b	=	visible surface crack extent

c1	=	empirical model constant 1 [1]

c2	=	empirical model constant 2 [1]

Dh	=	hydraulic diameter

da	=	maximum diameter of aggregate

f	=	Darcy-Weisbach friction factor

fc'	=	28-day specified concrete compressive strength

i	=	primary index

k	=	wall roughness coefficient, tertiary index

L	=	wall thickness

m	=	mass flow rate

n	=	turbulence parameter, number of items

nc	=	number of non-intersecting, visible cracks

P	=	absolute fluid pressure

P1	=	upstream fluid pressure

P2	=	downstream fluid pressure

Q	=	volumetric flow rate

Qm	=	experimental, measured volumetric flow rate

Qp	=	predicted volumetric flow rate

Qt	=	maximum allowable leakage rate [2]

Q1	=	volumetric flow rate on upstream side

Q2	=	volumetric flow rate on downstream side

qD	=	porous Darcy law component of the air flux vector field

qP	=	planar Poiseuille law component of the air flux vector field

Rs	=	specific gas constant for air

Re	=	Reynolds number

rp	=	prediction-to-measurement ratio

rp	=	average prediction-to-measurement ratio

s	=	sample standard deviation

S	=	visible frontal surface area

SC	=	Sutherland’s constant [3]

T	=	absolute temperature

T0	=	reference temperature for Sutherland’s law [3]

u	=	longitudinal (axial) velocity component

vavg.	=	average velocity

w	=	perpendicular crack width

wavg.	=	average perpendicular crack width

w0	=	closed effective crack width

y	=	transverse rectangular spatial dimension

Greek symbols:

α	=	crack-width-dependent empirical coefficient

β	=	crack-width-dependent empirical coefficient

γ	=	surface shear strain

∆P	=	difference in absolute pressure

ϵ	=	crack roughness height

θc	=	local crack inclination angle on surface

η0	=	empirical air flow exponent [4]

λ	=	empirically obtained friction factor

λB	=	friction factor [5]

λG	=	friction factor [6]

λGR	=	friction factor [7]

λS	=	friction factor [8]

μ	=	fluid dynamic viscosity

μ0	=	reference dynamic viscosity [3]

ρ	=	fluid mass density

ρl	=	longitudinal reinforcement ratio

ψ1	=	auxiliary quantity 1 for the Gelain [6] model

ψ2	=	auxiliary quantity 2 for the Gelain [6] model

ψ3	=	auxiliary quantity 3 for the Gelain [6] model

τ	=	shear stress on a surface

τ0	=	Darcy-Weisbach shear stress on a pipe wall

ξ	=	tortuosity of the crack network

Operators:

d	=	differential of a quantity

∆	=	difference, change in a quantity

∇ ⦁	=	divergence operator

Leakage rate prediction calculations

Sample calculations of leakage rates are presented here for select literature models. Measured geometry and flow rate data is sourced from the leakage rate test performed upon specimen L4, by Rizkalla et al. [9][10] at an applied tensile load of 45 kips (200 kN). Relevant input variables and properties of this test specimen are presented in the following table. The pressure at the downstream location, P2, is assumed to be at the standard atmospheric pressure of 101,325 Pa. Furthermore, crack measurements are taken at every non-intersecting, parallel crack. Since each crack is non-intersecting, the total flow rate is calculated by summing the flow rate contributions from each individual crack. The values of ∆P, b, L, and da for each crack are also assumed to be constant.

| Quantity | Value Expressed in US Customary Units | Value Expressed in SI Units |
| --- | --- | --- |
| b | 1 ft | 0.305 m |
| L | 0.583 ft | 0.1778 m |
| w1 | 2.63(10−4) ft | 8.00(10−5) m |
| w2 | 6.58(10−5) ft | 2.01(10−5) m |
| w3 | 1.31(10−4) ft | 3.99(10−5) m |
| w4 | 2.63(10−4) ft | 8.00(10−5) m |
| w5 | 1.31(10−4) ft | 3.99(10−5) m |
| w6 | 3.28(10−4) ft | 9.98(10−5) m |
| wavg. | 1.97(10−4) ft | 6.00(10−5) m |
| ∆P | 4,320 lb/ft2 | 206,800 Pa |
| P2 | 2,117 lb/ft2 | 101,325 Pa |
| P1 | 6,437 lb/ft2 | 308,125 Pa |
| R | 1,716 ft·lb/(slug·°R) | 287 J/(kg·K) |
| T | 530 °R | 292.26 K |
| μ | 3.80(10−7) lb·s/ft2 | 1.82(10−5) Pa·s |
| ρ | 0.00233 slug/ft3 | 1.2 kg/m3 |
| da | 0.0312 ft | 0.00953 m |

Planar Poiseuille incompressible flow model:

Qp=i=16∆Pbiwi312μLi
=b∆P12μL⋅i=16wi3
=0.3048206,800121.82⋅10-50.17782.15⋅10-12

=3.50⋅10-3m3s

Buss (1972) model:

Qp=i=16P12-P22P2biwi3300μLi
=b300μL⋅P12-P22P2⋅i=16wi3
=0.3048308,1252-101,32523001.82⋅10-5⋅0.1778101,3252.15⋅10-12

=5.65⋅10-4m3s

Rizkalla et al. (1984) model:

Using US customary units, the turbulence parameter (flow coefficient) n and the wall roughness coefficient k are calculated using the empirical power law relationships shown below. The summation of the cubed crack widths, ∑wi3, must be input with units of ft3.

n=0.1337.61⋅10-11-0.081
=0.878

k=2.907⋅107⋅7.61⋅10-110.428
=1,355

Proceeding,

Qp=bP22n+1RT1-nkμn⋅P12-P22Li=16wi312-n
=12,1171.51⋅1046.33⋅1077.61⋅10-110.891
=2.1610-2ft3s

=6.1110-4m3s

Suzuki et al. (1989) compressible flow model:

For each crack, the empirical coefficients a and β are calculated as follows. Typical calculations for one crack, w1, are shown.

a1=4.33⋅10-58⋅10-51.5+1
=61.5

β1=3.41⋅10-48⋅10-5
=4.26

The contribution from crack 1 is as follows:

Qp,1=2RTβ1⋅P12-P22P22⋅b12w13L1+6μb1RTa1P2β12-6μb1RTa1P2β1

Individual terms are calculated below.

c1=2RTβ1⋅P12-P22P22⋅b12w13L1
=167,7574.26⋅308,1252-101,3252101,3252⋅0.304828⋅10-530.1778
=8.69⋅10-8

c2=6μb1RTa1P2β1
=61.82⋅10-50.3048287292.2661.5101,3254.26
=3.98⋅10-4

Calculating the flow rate contribution,

Qp,1=8.69⋅10-8+3.98⋅10-42-3.98⋅10-4
=9.73⋅10-5m3s

Summing the flow rate contributions across all cracks,

Qp=i=16Qp,i
=9.73⋅10-5+8.75⋅10-7+…+1.87⋅10-4

=4.02⋅10-4m3s

Suzuki et al. (1992) incompressible model for slow gases:

Qp=i=1615.3wi+0.00756⋅∆Pbiwi3μLi
=b∆PμLi=1615.3wi+0.00756wi3
=0.3048206,8001.82⋅10-50.17781.91⋅10-14

=3.73⋅10-4m3s

Greiner and Ramm (1995) model:

The friction factor, λGR, is calculated using the nonlinear regression formula shown below. A typical calculation is presented for crack 1.

λGR,1=0.00177da0.409w10.575ln2415.46da+1.637da0.3043-0.024
=2.639⋅10-48⋅10-50.1833-0.373

=1.618

Qp=i=162RTλGR,i⋅P12-P22P22⋅bi2wi3Li
=b2RTL⋅P12-P22P22⋅i=16wi3λGR,i
=0.3048167,7570.17788.247⋅2.36⋅10-6

=2.01⋅10-3m3s

Badoux [5] model:

The friction factor calculation for crack 1 is shown below.

λB,1=4.73⋅10-4⋅da1.4w11.8+1
=4.73⋅10-4⋅0.0014824.22⋅10-8+1

=17.6

Qp=i=162RTλB,i⋅P12-P22P22⋅bi2wi3Li
=b2RTL⋅P12-P22P22⋅i=16wi3λB,i
=0.3048167,7570.17788.247⋅6.99⋅10-7

=5.94⋅10-4m3s

Gelain [6] model:

Upon solving the implicit relationship, the final expression is calculated as

Qp=i=16ψ2,i2-4ψ1,iψ3,i-ψ2,i2ρψ1,i

For crack 1, the expressions ψj,i are calculated as follows.

ψ1,1=2w1b
=28⋅10-50.3048
=0.0324

ψ2,1=μb2
=1.82⋅10-50.30482
=2.77⋅10-6

ψ3,1=-P12-P2248RT⋅b2w13L1+ξ
=-21,040⋅0.304828⋅10-530.17781.27
=-4.43⋅10-9

The contribution from crack 1 is then

Q1=2.77⋅10-62-40.0324-4.43⋅10-9-2.77⋅10-621.20.0324
=2.75⋅10-4m3s

The total contribution from all cracks is

Qp=i=16Qp,i
=2.75⋅10-4+1.86⋅10-5+…+3.76⋅10-4

=1.12⋅10-3m3s

References

[1]	G. Zakaib, “Pickering Unit #1 Containment Leakage Characterization,” Toronto, 1994. Accessed: Dec. 13, 2021. [Online]. Available: https://www.osti.gov/etdeweb/servlets/purl/612625

[2]	United States Nuclear Regulatory Commission, “Appendix J to Part 50—Primary Reactor Containment Leakage Testing for Water-Cooled Power Reactors.” [Online]. Available: https://www.nrc.gov/reading-rm/doc-collections/cfr/part050/part050-appj

[3]	W. Sutherland, “ LII. The viscosity of gases and molecular force ,” The London, Edinburgh, and Dublin Philosophical Magazine and Journal of Science, vol. 36, no. 223, pp. 507–531, Dec. 1893, doi: 10.1080/14786449308620508.

[4]	K. Okamoto, S. Hayakawa, and R. Kamimura, “Experimental Study of Air Leakage From Cracks in Reinforced Concrete Walls,” Nuclear Engineering and Design, vol. 156, pp. 59–165, 1995, Accessed: Dec. 13, 2021. [Online]. Available: https://www.sciencedirect.com/science/article/pii/002954939400941Q

[5]	M. Badoux, “SwissMetro - Air Permeability of Cracked Concrete Plates,” in 17th International Conference on Magnetically Levitated Systems and Linear Drives, Lausanne, 2002. Accessed: Mar. 18, 2023. [Online]. Available: http://www.maglev.ir/eng/documents/papers/conferences/maglev2002/topic5/IMT_CP_M2002_T5_S1_7.pdf

[6]	T. Gelain, “An Original Method to Assess Leakage Through Cracked Reinforced Concrete Walls,” Eng. Struct., vol. 38, pp. 11–20, May 2012, doi: 10.1016/j.engstruct.2011.12.037.

[7]	U. Greiner and W. Ramm, “Air Leakage Characteristics in Cracked Concrete,” Nuclear Engineering and Design, vol. 156, no. lY95, pp. 167–172, 1995, Accessed: Dec. 13, 2021. [Online]. Available: https://www.sciencedirect.com/science/article/pii/002954939400942R

[8]	T. Suzuki, K. Takiguchi, H. Hotta, N. Kojima, M. Fukuhara, and K. Kimura, “Experimental Study on the Leakage of Gas Through Cracked Concrete Walls,” in SMiRT-10, Anaheim: IASMiRT, Aug. 1989, pp. 145–150. Accessed: Dec. 13, 2021. [Online]. Available: https://repository.lib.ncsu.edu/bitstream/handle/1840.20/29858/DC_250663.pdf?sequence=1&isAllowed=y

[9]	S. H. Rizkalla, B. L. Lau, and S. H. Simmonds, “Air Leakage Characteristics in Reinforced Concrete,” Journal of Structural Engineering, vol. 110, no. 5, pp. 1149–1162, 1984, [Online]. Available: https://ascelibrary.org/doi/10.1061/%28ASCE%290733-9445%281984%29110%3A5%281149%29

[10]	B. L. Lau, “Study of Air Leakage Characteristics of Reinforced Concrete Containment Structures Subjected to High Internal Pressure,” University of Manitoba, Winnipeg, 1982. [Online]. Available: http://hdl.handle.net/1993/15085

Oh-Sung Kwon is a Professor and Associate Chair (Research) of Civil and Mineral Engineering at the University of Toronto, Canada. His research interests include the development of new experimental methods to improve understanding of structural behaviour subjected to various loading conditions.

Evan C. Bentz, FACI, is a Professor and Vice-Dean (Undergraduate) of Engineering at the University of Toronto. He received his BASc from the University of Waterloo, Canada in 1994 and his PhD from the University of Toronto in 2000. He is a member of ACI Committee 445, Shear and Torsion, and is former chair of Committee 365, Service Life Modelling. He received the ACI Chester Paul Seiss Award in 2018 as well as the ACI Design Award twice.